A Guarded Real-Robot Evaluation of a Fine-Tuned Vision-Language-Action Policy for Pick-and-Place

[Author Name], [Co-authors]

[Laboratory, Department, University]

# Abstract

This paper reports a controlled laboratory evaluation of a fine-tuned pi0.5-DROID vision-language-action policy on a Franka Research 3 robot. Physical execution depends on more than policy inference: action timing, camera synchronization, safety limits, grasp verification, and recovery behavior directly affect task completion. We collect synchronized RGB, robot-state, gripper-state, and action demonstrations; convert successful episodes to LeRobot format; fine-tune a joint-velocity policy; and execute it through a guarded reference controller. The system is evaluated on spatially held-out cube positions, with policy, control, gripper, and infrastructure outcomes reported separately. The study does not demonstrate construction-site deployment, construction-task completion, language generalization, or robustness to human interference. Instead, it establishes a controlled baseline for later component-level experiments involving contact, orientation, insertion, and tooling requirements.

# 1. Introduction

Construction automation motivates longer-term research on adaptable robot manipulation because object placement, visual context, material tolerances, and human activity can vary across tasks. This study does not reproduce those job-site conditions. It instead examines one controlled laboratory task that is relevant to developing and evaluating a real-robot learning pipeline.

Vision-language-action models combine visual observations, task language, and action prediction. They are commonly trained using robot demonstrations and imitation-learning objectives, often with pretrained vision-language representations. This study evaluates a fixed instruction and does not test language generalization, human-interference robustness, or adaptation to unstructured work environments.

This work evaluates the practical deployment of a fine-tuned pi0.5-DROID policy on a Franka Research 3 robot. The task is to pick a cube from a table position and place it in a basket within a fixed laboratory workspace. Although constrained, the task requires closed-loop perception, grasp timing, transport, release, and physical verification. The focus is whether policy actions can be executed reliably through the tested real-robot control stack, not whether the system is ready for construction deployment.

# 2. Research Positioning and Gap

Large VLA studies have demonstrated broad robot-control capabilities using extensive and diverse data. RT-1 evaluated language-conditioned control at scale, while RT-2 investigated transfer from web-scale vision-language learning to robot actions. OpenVLA made open VLA fine-tuning more accessible, and pi0/pi0.5 studied flow-based action generation and generalization to new tasks. These studies differ from the present controlled FR3 system in scale, robot embodiment, task, and deployment infrastructure.

Construction robotics has also moved beyond simple free-space pick-and-place. Prior work studies rebar assembly, pipe insertion, timber assembly, and force-aware imitation learning. This paper does not claim to advance construction-task performance directly. Its contribution is a controlled case study of adapting an open pi0.5-DROID policy to an FR3 robot and documenting the low-level requirements of the tested system: data collection, action representation, action-chunk execution, latency handling, workspace guards, physical grasp verification, and reset behavior.

# 3. Research Questions

RQ1. Under a fixed laboratory setup, can a fine-tuned pi0.5-DROID policy complete guarded closed-loop pick-and-place at spatially held-out table positions?

RQ2. Which control, timing, gripper, and safety mechanisms are required for reliable execution of policy action chunks on the FR3?

RQ3. Which observed failures should be attributed to policy prediction, physical grasping, control execution, or infrastructure faults?

RQ4 (future work). What additional sensing, tooling, demonstrations, and evaluation are required before testing contact-rich construction-component manipulation?

# 4. Contribution Claims

A laboratory FR3 demonstration pipeline that records RGB observations, robot and gripper state, actions, and metadata, then converts successful episodes to LeRobot format.

A fine-tuning and deployment pipeline for pi0.5-DROID using seven FR3 joint velocities and one gripper-closedness action at 15 Hz.

A guarded real-time execution design that separates model-predicted gripper-transition timing from physical gripper actuation and verifies grasp and release outcomes.

A controlled spatial-interpolation evaluation protocol with ten held-out positions and three planned trials per position.

A failure analysis framework that distinguishes policy failures from gripper, controller, safety-guard, and infrastructure failures.

Scope boundary: Add final success rates only after auditing the active paper evaluation ledger. Do not claim construction deployment, construction-task completion, language generalization, human-interference robustness, or robustness beyond the fixed laboratory setup from the cube-transfer experiment.

# 5. Current Experimental System

| Element | Current specification |
| --- | --- |
| Robot platform | Franka Research 3 with a parallel gripper. |
| Perception | Two Intel RealSense RGB cameras. |
| Training data | Successful real-robot demonstrations converted to LeRobot format. Confirm the final episode count from the released dataset metadata before submission. |
| Policy | Fine-tuned pi0.5-DROID checkpoint v3_12000. |
| Action representation | Seven joint-velocity values and one gripper-closedness value. |
| Execution | 15 Hz timestamped receding-horizon streaming with a reference controller, latency checks, workspace guards, and physical gripper verification. |
| Evaluation protocol | Ten spatially held-out positions, I01-I10, with three planned trials per position. |

# 6. Future Work: Contact-Rich Construction-Component Manipulation

The cube-transfer task is a necessary commissioning benchmark, but it is predominantly free-space manipulation. The next phase should use the supplied component geometries: rslScrew.stl, rsl_flat_brkt.stl, and main-1(1).stl. Their exact physical role, dimensions, material, tolerances, fixture design, and required tools must be documented before data collection.

| Stage | Task | What it measures | Additional requirement |
| --- | --- | --- | --- |
| 1 | Grasp and orient the bracket. | Geometry-aware grasping and pose control. | Repeatable fixture and pose labels. |
| 2 | Place the bracket in a fixture or mating location. | Pose accuracy and approach planning. | Known placement tolerance and success criterion. |
| 3 | Pick, orient, and insert the screw into a fixture. | Small-object manipulation and contact onset. | Reliable screw grasp and compliant insertion strategy. |
| 4 | Fastening or tightening. | True construction assembly and torque-sensitive contact. | A suitable driver end effector, torque/force sensing or estimation, and safety limits. |

A parallel gripper alone can support early grasping and insertion experiments, but it cannot validate a true screw-fastening task. Fastening should therefore be treated as a later experiment after adding an appropriate tool and contact-sensing or compliance strategy.

# 7. Experimental Design for the Next Stage

Measure and document component geometry, mass, graspable surfaces, fixture geometry, and tolerances.

Commission scripted motion and a recorded-action replay before enabling policy control.

Collect demonstrations for each task phase: approach, grasp, transport, alignment, insertion, and release or fastening.

Define separate train, validation, and held-out component poses before collection to avoid leakage.

Evaluate at least a scripted baseline and a task-specific behavior-cloning baseline in addition to the fine-tuned VLA.

Report grasp success, placement or insertion success, completion time, safety stops, intervention count, and failure category.

# 8. Literature Fact Check

Do not write that previous VLA or construction-robotics papers only test simple pick-and-place. pi0 and pi0.5 report dexterous and long-horizon tasks, while construction robot-learning studies include rebar assembly, pipe insertion, and force-aware assembly. A defensible statement is that many influential VLA benchmarks focus on general manipulation, whereas the proposed next-stage benchmark isolates construction-component geometry, contact, tolerance, and fastening requirements on the same FR3 platform.

# 9. References

Black, K., et al. pi0: A Vision-Language-Action Flow Model for General Robot Control. 2024. https://arxiv.org/abs/2410.24164

Black, K., et al. pi0.5: A Vision-Language-Action Model with Open-World Generalization. CoRL, 2025. https://proceedings.mlr.press/v305/black25a.html

Kim, M. J., et al. OpenVLA: An Open-Source Vision-Language-Action Model. 2024. https://arxiv.org/abs/2406.09246

Brohan, A., et al. RT-1: Robotics Transformer for Real-World Control at Scale. 2022. https://arxiv.org/abs/2212.06817

Brohan, A., et al. RT-2: Vision-Language-Action Models Transfer Web Knowledge to Robotic Control. 2023. https://arxiv.org/abs/2307.15818

Sun, T., et al. Mobile Robotic Rebar Cage Assembly via Imitation Learning. Automation in Construction, 2026. https://www.sciencedirect.com/science/article/pii/S0926580525007113

You, H., et al. Force-Based Robotic Imitation Learning: A Two-Phase Approach for Construction Assembly Tasks. 2025. https://arxiv.org/abs/2501.14942

Wang, Y., et al. Robotic Imitation of Human Assembly Skills Using Hybrid Trajectory and Force Learning. 2021. https://arxiv.org/abs/2103.05912